| GPIO | Input | Output | Notes |  | GPIO | Input | Output | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Pulled up | OK | Outputs PWM signal at boot |  | 16 | OK | OK |  |
| 1 | TX pin | OK | Debug output at boot |  | 17 | OK | OK |  |
| 2 | OK | OK | Connected to on-board LED |  | 18 | OK | OK |  |
| 3 | OK | RX pin | HIGH at boot |  | 19 | OK | OK |  |
| 4 | OK | OK |  |  | 21 | OK | OK |  |
| 5 | OK | OK | Outputs PWM signal at boot |  | 22 | OK | OK |  |
| 6 | x | x | Connected to the integrated SPI flash |  | 23 | OK | OK |  |
| 7 | x | x | Connected to the integrated SPI flash |  | 25 | OK | OK |  |
| 8 | x | x | Connected to the integrated SPI flash |  | 26 | OK | OK |  |
| 9 | x | x | Connected to the integrated SPI flash |  | 27 | OK | OK |  |
| 10 | x | x | Connected to the integrated SPI flash |  | 32 | OK | OK |  |
| 11 | x | x | Connected to the integrated SPI flash |  | 33 | OK | OK |  |
| 12 | OK | OK | Boot fail if pulled high |  | 34 | OK |  | input only |
| 13 | OK | OK |  |  | 35 | OK |  | input only |
| 14 | OK | OK | Outputs PWM signal at boot |  | 36 | OK |  | input only |
| 15 | OK | OK | Outputs PWM signal at boot |  | 39 | OK |  | input only |

Wire пины по умолчанию

GPIO22 Wire SCL

GPIO21 Wire SDA